# Connecting the G-Sat External GPS Device

## Overview

This document includes configuration information related to connecting to the G-SAT BT-368 Bluetooth GPS Navigation Receiver.

The G-SAT BT-368 Unit looks like:

## Introduction

Use this document to configure the following options:

- Turning On/Off the Device

- Auto Shut-Off Information

- LED Status Indicators

- Pairing With Handhelds

  - Windows Mobile 5.x

  - Windows CE/.Net

  - Windows Mobile 6.x (coming soon)

## Turning On/Off the Device

The BT-368 GPS receiver has an On/Off button located on the top of the unit. To turn the unit on, press and hold the On/Off button until the Power LED becomes lit. To turn off the unit, repeat the process and the Power LED will turn off.

## Auto Shut-Off

The BT-368 GPS receiver is equipped with an auto shut-off power saving feature. To conserve the battery life, whenever the GPS receiver doesn’t detect a Bluetooth signal for over 10 minutes the receiver will automatically shut off unless its connected to an external power source (AC or DC Charger).

## LED Status Indicators

The G-SAT BT-368 External GPS device has a small LED status indicator light on the unit. The section below describes what the different color lights indicate:

### Power LED Definitions:

RED: Battery Power Low (Needs Charging)

YELLOW:  Battery Currently Charging

LED OFF: Battery Charged

### GPS LED Definitions:

STEADY ON: No GPS connection with Satellite Signals

FLASHING: GPS connected and receiving signals

### Bluetooth LED Definitions:

SLOW FLASHING: Not connected to a Bluetooth Device

FAST FLASHING: Connected with a Bluetooth Device

# Pairing the Device with Handhelds

## Introduction

The following sections will instruct the users how to configure the G-SAT BT-368 unit with the most common handhelds/operating systems.

## Windows Mobile 5.x (Dell/M3 Style)

Setup Pairing:
1) On the Dell, tap Start->Settings->Connections Tab->Bluetooth
2) Ensure "Turn on Bluetooth" is checked
3) Tap the Devices tab
4) Ensure the Bluetooth GPS is turned on
4) Tap "New Partnership..."
5) It will then scan for the Bluetooth GPS and should show up as "BT-GPS-XXXXXX" (The X's will be random)
6) Tap the "BT-GPS-XXXXXX" and select the Next button
7) Enter 0000 for the Passkey and tap the Next button
8) Check the "Serial Port" from the "Select services to use from this device." list and then select the Finish button.
9) Tap on the "COM Ports" tab
10) Select the "New Outgoing Port" item
11) Tap the "BT-GPS-XXXXXX" and select the Next button
12) Uncheck the "Secure Connection" and make note of the COM port (should be COM7)
13) Tap Finished button
14) You are now paired with the Bluetooth GPS device.

*NOTE* If on step 8 you do not see the "Serial Port" item then power off the Bluetooth GPS device, reset the HH, and start again from Step 1.

Setup Connection (Direct Method):
1) In RMA, open the Handheld Class screen in edit mode for the class you're wanting to change
2) Select the Hardware tab and ensure the following settings are set:
Bluetooth GPS = Checked
Log GPS Info on Handheld = Checked
GPS Baud Rate = 48000
GPS COM Port = COM7: (this will be the COM port defined on Step 12 of the "Setup Paring")
Logging Interval (in seconds) = 10 (this is whatever you prefer)
3) On the Sales Rules tab, you can also check "Show GPS location button" to allow capture of customer coordinates from the Confirm Sale screen.
4) Create your Morning Card and the HH will now use the GPS 

Setup Connection (Managed Method):
1) On the Dell, tap Start->Settings->System->GPS
2) On the Programs tab, choose COM8 for the "GPS program port:"
3) On the Hardware tab, choose COM7 for the "GPS hardware port:" (this will be the COM port defined on Step 12 of the "Setup Paring").
4) On the Hardware tab, also select 4800 for the "Baud rate:"
5) On the Access tab, ensure "Manage GPS automatically" is checked and tap OK.
6) In RMA, open the Handheld Class screen in edit mode for the class you're wanting to change
7) Select the Hardware tab and ensure the following settings are set:
Bluetooth GPS = Checked
Log GPS Info on Handheld = Checked
GPS Baud Rate = 48000
GPS COM Port = COM8: (this will be the COM port defined on Step 2 of the "Setup Connection (Managed Method)")
Logging Interval (in seconds) = 10 (this is whatever you prefer)
8) On the Sales Rules tab, you can also check "Show GPS location button" to allow capture of customer coordinates from the Confirm Sale screen.
9) Create your Morning Card and the HH will now use the GPS.

## Windows CE/.NET (MangoGT and GX Style)

Setup Pairing:
1. On the Device, tap Start button/Windows logo ->Settings, then choose Bluetooth Device Properties
2. Ensure "Enable Bluetooth Device" is checked
3. Tap the Scan Device Service Button
4. Tap the Scan Device button
5. There will likely be TWO BT-GPS-XXXXXX devices listed.
6. Double Tap one of the BT-GPS-XXXXXX and choose “Trusted”
7. Select “Yes” when prompted “Do you need to authenticate the device”
8. Enter 0000 for the Bluetooth Pin and tap the OK button
9. Double Tap the OTHER BT-GPS-XXXXXX and choose “Trusted”
10. Select “Yes” when prompted “Do you need to authenticate the device”
11. Enter 0000 for the Bluetooth Pin and tap the OK button
12. Select the OK button in the upper right corner of the “Bluetooth Manager” screen.
13. Select the "Serial Port" tab.
14. From the dropdown, choose COM6 (make note of this, it will be necessary to enter this value in RMA setup)
15. Select the BT-GPS-XXXXXX option under the Device Name box
16. Select the “Register Serial Port” button
17. Select the OK button in the upper right corner of the “Bluetooth Manager” screen.
18. You are now paired with the Bluetooth GPS device.
19. Proceed to setting up RMA Handheld Class settings (as directed in “Setup Connection (Direct Method) below).

*NOTE* If on step 15 you do not see the “BT-GPS-XXXXXX” device listed then power off the Bluetooth GPS device, reset the HH, and start again from Step 1.

## Windows Mobile 6.x (Coming Soon)

This section will be coming soon.